| PN (PN): |  |  | SN: |  |  |  |  | Nº da OS (WO): |  |  | TLN: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LM200SA47 |  |  | N/C |  |  |  |  | 038077 |  |  | 2686 |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  |  |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço PrincipaL (Main Service) |  | QUANT: |  |  |
| LMG4569-058 |  |  |  | REAR MOUNTS |  |  | REVISÃO GERAL / OVERHAUL |  | 8 |  |  |
| TSN: |  |  |  | CSN: |  |  | TSO: |  | CSO: |  |  |
| ITEM (Item) | TAREFA (Task) | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |  |  |  |  |
| 1 | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | EXECUTAR LIMPEZA CONFORME IP006 - ITEM 3.35 COM REMOÇÃO DO CADMIO. PERFORM CLEANING IN ACCORDANCE WITH IP006 - ITEM 3.35 WITH CADMIUM REMOVAL. |  |  |  |  |  |  |
| 2 | CHEMICAL CLEANING | END- LIQUIDO PENETRANTE / NDT - FLUORESCENT PENETRANT INSPECTION |  |  | REALIZAR INSPEÇÃO DE NDT – FPI NORMA ASTM 1417, LEVEL 2, TIPO 1, METODO A. PERFORM NDT INSPECTION – FPI ASTM STANDARD 1417, LEVEL 2, TYPE 1, METHOD A. FORAM ENCONTRADAS INDICAÇÕES? WERE INDICATIONS FOUND? ( ) SIM / YES – VER IAS RG-0332 ANEXO / SEE IAS RG-0332 ANNEX ( ) NÃO / NO |  |  |  |  |  |  |
| 3 | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | REALIZAR LIMPEZA PÓS NDT CONFORME IP-006, ITEM 3.34. PERFORM POST-NDT CLEANING IN ACCORDANCE WITH IP-006, ITEM 3.34. |  |  |  |  |  |  |
| 4 | T56A | INSPEÇÃO / INSPECTION |  |  | REALIZAR INSPEÇÃO APÓS OPERAÇÃO DE FPI. PERFORM INSPECTION AFTER FPI OPERATION. REFIRA-SE AOS RESULTADOS REGISTRADOS NO IAS RG-0332 (CASO TENHA) E PREENCHER O MESMO COM DEVIDAS OBSERVAÇÕES. REFER TO THE RESULTS RECORDED IN IAS RG-0332 (IF ANY) AND FILL IT OUT WITH APPROPRIATE OBSERVATIONS AVALIAR OS LIMITES SE ACEITAÇÃO CONFORME OHM EVALUATE THE LIMITS OF ACCEPTANCE ACCORDING TO OHM ( ) APROVADO / APPROVED ( ) REJEITADO / REJECTED ANEXAR E REGISTRAR Nº LAUDO NDT:________________ ATTACH AND REGISTER NDT REPORT NO. |  |  |  |  |  |  |
| 5 | REPAIR SHOP | PINTURA / PAINTING |  |  | ISOLAR AREAS ADJACENTES (BORRACHAS E FUROS DOS LORDS) E APLICAR PRIMER NAS PARTES CONFORME IP-0005 ITEM 3.6 5ISOLATE ADJACENT AREAS (RUBBERS AND HOLES IN LORDS) AND APPLY PRIMER TO THE PARTS IN ACCORDANCE WITH IP-0005 ITEM 3.6 |  |  |  |  |  |  |
| 6 | CHEMICAL CLEANING | PINTURA / PAINTING |  |  | ISOLAR AREAS ADJACENTES (BORRACHAS / FACE TRASEIRA DE CONTATO DOS LORDS E FUROS) E APLICAR Aluminized-epoxy enamel NAS OUTRAS AREAS DEMARCADAS CONFORME IP-0005 ITEM 3.6 ISOLATE ADJACENT AREAS (RUBBERS / REAR SIDE OF CONTACT OF LORDS AND HOLES) AND APPLY Aluminized-epoxy enamel IN THE OTHER AREAS DEMARCED IN ACCORDANCE WITH IP-0005 ITEM 3.6 |  |  |  |  |  |  |
| 7 | REPAIR SHOP | PINTURA / PAINTING |  |  | REMOVER ISOLAÇÕES E EXCESSOS, LIMPAR COM AR COMPRIMIDO REMOVE INSULATION AND EXCESS, CLEAN WITH COMPRESSED AIR |  |  |  |  |  |  |
| 8 | T56A | INSPEÇÃO / INSPECTION |  |  | INSPEÇÃO FINAL FINAL INSPECTION |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
| OBSERVAÇÕES (Observation): |  |  |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA (Date): |  |
|  |  |  |  |  |  | ANDRÉ |  |  |  | 17/12/24 |  |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department)