| PN (PN): |  |  | SN: |  |  |  |  | Nº da OS (WO): |  |  | TLN: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6841213 |  |  | RR4797 |  |  |  |  | PRINCIPAL |  |  | 2188 |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  |  |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço PrincipaL (Main Service) |  | QUANT: |  |  |
| AE-109692 / IAS |  |  |  | Wheel 3rd Stage |  |  | REPARO / REPAIR |  | 1 |  |  |
| TSN: |  |  |  | CSN: |  |  | TSO: |  | CSO: |  |  |
| ITEM (Item) | TAREFA (Task) | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |  |  |  |  |
|  | Manual Repair | REPAROS MANUAIS / MANUAL REPAIR |  |  | Suavizar rebarbas nos furos dos pinos na área de instalação das blades e danos nas áreas de encaixe. Smooth out burrs in the pin holes in the blade installation area and damage in the fitting areas. |  |  |  |  |  |  |
|  | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | Limpar conforme IP-0006, 3.23 Clean per IP-0006, 3.23 |  |  |  |  |  |  |
|  | CHEMICAL CLEANING | NON-DESTRUCTIVE TESTING |  |  | Executar MPI: ASTM e 1444. Apresentou indicação? ( )Sim ( )Não Run MPI: ASTM E 1444. Did you present an indication? ( )Yes ( )No |  |  |  |  |  |  |
|  | CHEMICAL CLEANING | NON-DESTRUCTIVE TESTING |  |  | Executar FPI: ASTM e 1417, Method-D, Level-4. Apresentou indicação? ( )Sim ( )Não Run FPI: ASTM E 1444, Method-D, Level-4 Did you present an indication? ( )Yes ( )No |  |  |  |  |  |  |
|  | T56A | INSPEÇÃO / INSPECTION |  |  | Realizar inspeção pós NDT conforme. (Perform inspection after NDT I.A.W. OHM). Refira-se aos resultados registrados no RG-0332 (se foi feito na etapa anterior) e preencher o mesmo com devidas observações. (Refer to the results registered in the RG-0332 (if it was done in the step before) and fill it in with due observations). |  |  |  |  |  |  |
|  | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | Realizar limpeza pós NDT conforme IAS-IP-0006 Item 3.34 Rev.03. Perform Cleaning Post-NDT I.A.W. IAS-IP-0006 Item 3.34 Rev.03. |  |  |  |  |  |  |
|  | CHEMICAL CLEANING | JATEAR / BLAST |  |  | Jatear utilizando Glass Bead para limpeza da roda conforme IP-0006, 3.26 Blast using Glass Bead to clean of the wheel per IP-0006, 3.26 |  |  |  |  |  |  |
|  | CHEMICAL CLEANING | DEPOSIÇÃO DE NIQUEL-CADMIO / NICKEL-CADMIUM PLATING |  |  | Aplicar banho de nickel cádmio conforme TASK 70-01-31-300-801 Apply nickel cadmium plating, per TASK 70-01-31-300-801 |  |  |  |  |  |  |
|  | T56A | INSPEÇÃO / INSPECTION |  |  | INSPEÇÃO FINAL. ( )APROVADO ( )REPROVADO FINAL INSPECTION. ( )APPROVED ( )FAILED |  |  |  |  |  |  |
| OBSERVAÇÕES (Observation): |  |  |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA (Date): |  |
|  |  |  |  |  |  | Kauã Fernandes |  |  |  | 18/12/2024 |  |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department)