| PN(PN): |  | SN: | ARTIGO/PARTE(Article/Part): |  |  | QUANT: |  | TLN: |  | Nº da OS(WO): |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 393291-21 |  | LMG3269-16 | QEC ASSY |  |  | 1 |  | T.O 1C-130A-36 |  | 040128 |
| ITEM (Item) | DESCRIÇÃO DO SERVIÇO A EXECUTAR (Description of thework to be executed) |  |  | EXECUÇÃO (Execution) |  |  | *SETOR/FORNECEDOR (Sector/Supplier) |  |  |  |
|  | EXECUTAR LIMPEZA E DECAPAGEM DE TODAS AS PARTES DO Q.E.C CONFORME IP006, ITENS 3.35 E 3.18 PERFORM CLEANING AND STRIPPING OF ALL PARTS OF THE Q.E.C. IN ACCORDANCE WITH IP006, ITEMS 3.35 AND 3.18 |  |  |  |  |  | LIMPEZA |  |  |  |
|  | REALIZAR EDDY CURRENT INSPECTION NAS SEGUINTES AREAS ABAIXO CONFORME T.O 1C-130A-36 (ANEXA): PERFORM EDDY CURRENT INSPECTION IN THE FOLLOWING AREAS BELOW IN ACCORDANCE WITH T.O 1C-130A-36 (ATTACHED): 6-11. – INSPECTION N-2 (AREAS A, B E C) : QEC LONGERONS, FIREWALL FRAME FORGING AND FIREWALL ASSEMBLY (FIGURA 6-2) 6-27A. – INSPECTION N-5 (GEARBOX ALIGNMENT HOLE) : QEC LOWER LONGERON RADIUS AREAS (FIGURA 6-3) 6-28. – INSPECTION N-6 (AREAS A, B, C e D) : AFT ENGINE MOUNT BEAM AND AFT ENGINE MOUNT BEAM SUPPORTS (FIGURA 6-4, SHEET 1 E 2) FORAM ENCONTRADAS INDICAÇÕES? INDICATIONS WERE FOUND ( ) SIM – VER IAS RG-0332 ANEXO YES – SEE IAS RG-0332 ANNEX ( ) NÃO NO OBS: FAZER LAUDO SEPARADO PARA CADA AREA INSPECIONADA. NOTE: MAKE A SEPARATE REPORT FOR EACH INSPECTED AREA. |  |  |  |  |  | NDT IAS |  |  |  |
|  | REALIZAR LIMPEZA PÓS NDI CONFORME IP-006, ITEM 3.34. PERFORM POST-NDI CLEANING IN ACCORDANCE WITH IP-006, ITEM 3.34. |  |  |  |  |  | LIMPEZA |  |  |  |
| 1 | REALIZAR INSPEÇÃO APÓS OPERAÇÃO DE NDI. PERFORM INSPECTION AFTER NDI OPERATION REFIRA-SE AOS RESULTADOS REGISTRADOS NO IAS RG-0332 (CASO TENHA) E PREENCHER O MESMO COM DEVIDAS OBSERVAÇÕES. REFER TO THE RESULTS RECORDED IN IAS RG-0332 (IF ANY) AND FILL IT OUT WITH APPROPRIATE OBSERVATIONS. AVALIAR OS LIMITES SE ACEITAÇÃO CONFORME MANUAL T.O 1C-130A-36 EVALUATE ACCEPTANCE LIMITS IN ACCORDANCE WITH MANUAL T.O 1C-130A-36 ( ) APROVADO / APPROVED ( ) REJEITADO / REJECTED AREA N-2 - ANEXAR N° LAUDO NDI:________________ ATTACH NDI REPORT NO. AREA N-5 - ANEXAR N° LAUDO NDI:________________ ATTACH NDI REPORT NO. AREA N-6 - ANEXAR N° LAUDO NDI:________________ ATTACH NDI REPORT NO. |  |  |  |  |  | INSPEÇÃO |  |  |  |
| 3 | INSPEÇÃO FINAL FINAL INSPECTION. |  |  |  |  |  | INSPEÇÃO |  |  |  |
| OBSERVAÇÕES (Observation): |  |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA (Date): |  |
| T.O. 1C-130A-36 - SECTION VI NACELLE GROUP NONDESTRUCTIVE INSPECTION (NDI) OF NACELLE. 6-11. – INSPECTION N-2 6-27A. – INSPECTION N-5 6-28. – INSPECTION N-6 |  |  |  |  | Leonardo S Gomes |  |  |  | 13/01/2025 |  |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department)