| PN: |  |  |  | SN: |  |  |  |  | Nº DA OS (WO): |  |  |  | TLN: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6899400 |  |  |  | CAE-270882 |  |  |  |  | 37202 |  |  |  | 2118 |  |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  |  |  |  | ARTIGO/PARTE (Article/Part): |  |  |  |  | SERVIÇO PRINCIPAL (Main Service): |  |  |  | QUANT: |
| M250-C20J / CAE-270882 |  |  |  |  | ENGINE ASSY |  |  |  |  | REPARO / REPAIR |  |  |  | 01 |
| TSN: |  |  | CSN: |  |  |  | TSO: |  |  |  | CSO: |  |  |  |
| Conforme Heavy Maintenance Manual _ CSP22005 _ Rev. 9, Jun. 15/2020 (According to Heavy Maintenance Manual _ CSP22005 _ Rev. 9, Jun. 15/2020) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ITEM (Item) | TAREFA (Task) | ETAPA (Step) |  |  |  | DESCRIÇÃO (Description) |  |  |  |  |  |  |  |  |
| 01 | M250 | DESMODULAÇÃO / DESMODULATION |  |  |  | Realizar desmodulação dos módulos de turbina e compressor, conforme OHM 10W3 R16. Perform the demodulation of the turbine and compressor module, as per OHM 10W3 R16. |  |  |  |  |  |  |  |  |
| 02 | M250 | INSPEÇÃO / INSPECTION |  |  |  | Verificar todas as panes reportas pelo banco de provas no RG-0007 na O.S 37202. Check all faults reported by the test bench in RG-0007 in O.S 37202 |  |  |  |  |  |  |  |  |
| 03 | M250 | DESMONTAGEM / DISASSEMBLY |  |  |  | Realizar desmontagem do modulo de compressor conforme OHM 10W3 R16. Perform disassembly of the module compressor as per OHM 10W3 R16. |  |  |  |  |  |  |  |  |
| 04 | M250 | BALANCEAMENTO / BALANCING |  |  |  | Realizar balanceamento do rotor de compressor e preencher formulário FS. 0704 e registrar valor final do balanceamento. Perform balance of compressor rotor, fill form FS. 0704 and record final values of balancing. Values: Final LH: _________ Final RH: __________ NCC: ___________ Valid until: ___________. |  |  |  |  |  |  |  |  |
| 05 | M250 | MONTAGEM / ASSEMBLY |  |  |  | Realizar montagem do módulo de compressor utilizando como referência o OHM - 10W3 e registrando as informações no formulário FS.0677. Assembly the compressor module per OHM - 10W3 as a reference and recording the information on form FS.0677. |  |  |  |  |  |  |  |  |
| 06 | M250 | MODULAÇÃO / MODULATION |  |  |  | Realizar modulação do módulo de compressor, conforme HMM CSP22005. Perform the modulation of the compressor module, as per HMM CSP22005. |  |  |  |  |  |  |  |  |
| 07 | M250 | MODULAÇÃO / MODULATION |  |  |  | Realizar modulação do módulo de turbina, conforme HMM CSP22005. Perform the modulation of the turbine module, as per HMM CSP22005. |  |  |  |  |  |  |  |  |
| 08 | Banco de provas | TESTE /TEST |  |  |  | Realizar teste do motor, conforme HMM CSP22005, verificando performance, vibração, vazamentos em geral e ruídos anormais, efetuando os registros necessários no formulário FS.1041. Perform the engine test, as per HMM CSP22005, checking performance, vibration, general leaks, and abnormal noises, making the necessary entries in the FS.1041 form. |  |  |  |  |  |  |  |  |
| 09 | Banco de provas | PRESERVAÇÃO / PRESERVATION |  |  |  | Realizar preservação do motor, conforme HMM CSP22005. Perform engine preservation, as per HMM CSP22005. |  |  |  |  |  |  |  |  |
| 10 | M250 | ACABAMENTO / FINISHING |  |  |  | Realizar acabamento do motor, efetuando os registros necessários no formulário FS.0787: - Verificar todos os acessórios; - Verificar se todos os processos solicitados foram cumpridos; - Verificar se todo o material recebido está sendo enviado de volta; - Verificar e assinar todas as etapas do formulário FS.0805 nas páginas 4 e 5; - Verificar a necessidade de despacho dos materiais sucata. Perform engine finishing, making the necessary entries in the FS.0787 form. - Verify all accessories; - Verify if all requested processes have been completed; - Verify if all received materials are being sent back; - Verify and sign all steps of the FS.0805 form on pages 4 and 5; - Verify the need for disposal of scrap materials. |  |  |  |  |  |  |  |  |
| 11 | M250 | INSPEÇÃO / INSPECTION |  |  |  | Realizar inspeção final, conforme HMM CSP22005. Perform final inspection, as per HMM CSP22005. |  |  |  |  |  |  |  |  |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department)