| PN (PN): |  |  | SN: |  |  |  |  | Nº da OS (WO): |  |  | TLN:02188 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6848669 |  |  | XX14221 |  |  |  |  | IAS |  |  |  |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  |  |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço PrincipaL (Main Service) |  | QUANT: |  |  |
| IAS / AE-109696 |  |  |  | AIR INLET HOUSING |  |  | REVISÃO GERAL / OVERHAUL |  | 1 |  |  |
| ITEM (Item) | TAREFA (Task) | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |  |  |  |  |
| 1 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | REMOVER O CAGE CENTRAL. REMOVE THE CENTER CAGE. |  |  |  |  |  |  |
| 2 | CHEMICAL CLEANING | DEPOSIÇÃO DE CROMO / CHROME PLATING |  |  | APLICAR 0.006¨DE ESPESSURA DE CROMO DURO NO DIÂMETRO INTERNO DO CAGE. APPLY 0.006 ̈THICK HARD CHROME TO THE INNER DIAMETER OF THE CAGE. |  |  |  |  |  |  |
| 3 | CHEMICAL CLEANING | TRATAMENTO TERMICO / HEAT TREATMENT |  |  | TRATAMENTO TÉRMICO NO CAGE CROMADO EM 190° C POR 3- HORAS. HEAT TREATMENT IN THE CHROME CAGE AT 190° C FOR 3- HOURS. |  |  |  |  |  |  |
| 4 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | MONTAR O CAGE E FRENAR COM 2- PINOS P/N= MS9105-53 MOUNTING THE CAGE AND BRAKING WITH 2-PIN P/N= MS9105-53 |  |  |  |  |  |  |
| 5 | REPAIR SHOP | RETIFICA / RECTIFY |  |  | RETIFICAR O DIÂMETRO INTERNO DO CAGE CROMADO, DEIXANDO= 4.3340¨+ou-0.0002¨ CENTRAR PELO DIÂMETRO EXTERNO DA FACE ADJACENTE. REMOVER CANTO VIVO. RECTIFY THE INNER DIAMETER OF THE CHROME CAGE, ALLOWING= 4.3340 ̈+or-0.0002 ̈ CENTER BY THE OUTSIDE DIAMETER OF THE ADJACENT FACE. REMOVE LIVE CORNER. |  |  |  |  |  |  |
| 6 | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | EXECUTAR LIMPEZA CONFORME A IP-0006, 3.10 PERFORM CLEANING IN ACCORDANCE WITH IP-0006, 3.10 |  |  |  |  |  |  |
| 7 | NDT | END- LIQUIDO PENETRANTE / NDT - FLUORESCENT PENETRANT INSPECTION |  |  | EXECUTAR FPI: ASTM E 1417, METHOD-A, LEVEL 3. APRESENTOU INDICAÇÃO? ( )SIM ( )NÃO RUN FPI: ASTM E 1417, METHOD-A, LEVEL 3. DID YOU PRESENT AN INDICATION? ( )YES ( )NO |  |  |  |  |  |  |
| 8 | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | LIMPAR CONFORME A IP-0006, 3.10 CLEAR AS PER IP-0006, 3.10 |  |  |  |  |  |  |
| 9 | REPAIR SHOP | ISOLAÇÃO / MASK |  |  | ISOLAR OS PRISIONEIROS, INSERTS E ÁREAS DE CONTATO. ISOLATE STUDS, INSERTS AND CONTACT AREAS. |  |  |  |  |  |  |
| 10 | REPAIR SHOP | PINTURA / PAINTING |  |  | PINTAR COM: PRIMER PA-0060 E CINZA PA-0065. PAINT WITH: PRIMER PA-0060 AND GRAY PA-0065. |  |  |  |  |  |  |
| 11 | REPAIR SHOP | REMOVER ISOLAMENTO / REMOVE MASK |  |  | REMOVER AS ISOLAÇÕES E OS EXCESSOS DE TINTA. REMOVE INSULATION AND EXCESS PAINT. |  |  |  |  |  |  |
| 12 | REPAIR SHOP | PINTURA / PAINTING |  |  | ISOLAR E APLICAR EMBORRACHAMENTO NO BORDO DE ATAQUE DOS STRUTS. ISOLATE AND APPLY RUBBER COATING TO THE LEADING EDGE OF THE STRUTS. |  |  |  |  |  |  |
| 13 | T56A | INSPEÇÃO / INSPECTION |  |  | INSPEÇÃO FINAL. Lubrificar os prisioneiros. ( )APROVADO ( )REPROVADO FINAL INSPECTION. ( )APPROVED ( )FAILED |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
|  | Escolher um item. | Escolher um item. |  |  |  |  |  |  |  |  |  |
| OBSERVAÇÕES (Observation): |  |  |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA (Date): |  |
|  |  |  |  |  |  | VILSON BARBOSA |  |  |  | 20/08/2025 |  |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department)