| PN (PN): |  |  | SN: |  |  |  |  | Nº da OS (WO): |  |  | TLN:02188 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23006940 |  |  | AA2811 |  |  |  |  | 020250 |  |  |  |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  |  |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço PrincipaL (Main Service) |  | QUANT: |  |  |
| AE- 109692 |  |  |  | ACESSORY BOX HOUSING |  |  | REVISÃO GERAL / OVERHAUL |  | 1 |  |  |
| ITEM (Item) | TAREFA (Task) | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |  |  |  |  |
| 1 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | A-) REMOVER OS DOIS CAGES CENTRAIS ASSINALADOS. B-) INSTALAR OS 3- PRISIONEIROS, DEIXANDO A ALTURA= 1.828¨ C-) PASSAR A BROCA FERRAMENTA NOS 4- FUROS PASSANTES. A-) REMOVE THE TWO CENTRAL CAGES MARKED. B-) INSTALL THE 3- PRISONERS, LEAVING THE HEIGHT = 1.828 ̈ C-) PASS THE TOOL DRILL IN THE 4-THROUGH HOLES. |  |  |  |  |  |  |
| 2 | REPAIR SHOP | USINAGEM / MACHINING |  |  | USINAR OS DOIS ALOJAMENTOS DOS CAGES CENTRAIS REMOVIDOS, CENTRANDO PELOS PROPRIOS DIÂMETROS INTERNOS, ATÉ DESOVALIZA-LOS. PODE FICAR PEQUENA MANCHA. MACHINE THE TWO REMOVED CENTRAL CAGE HOUSINGS, CENTERING BY THEIR OWN INTERNAL DIAMETERS, UNTIL THEY ARE DEVALUED. IT MAY BE A SMALL STAIN. |  |  |  |  |  |  |
| 3 | T56A | INSPEÇÃO / INSPECTION |  |  | CRIAR O.S. PARA CROMAR OS CAGES, JÁ DETERMINANDO A MEDIDA FINAL DOS MESMOS. CREATE O.S. TO CHROME THE CAGES, ALREADY DETERMINING THEIR FINAL MEASUREMENT. |  |  |  |  |  |  |
| 4 | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | LIMPAR CONFORME A IP-0006, 3.10 CLEAR AS PER IP-0006, 3.10 |  |  |  |  |  |  |
| 5 | NDT | END- LIQUIDO PENETRANTE / NDT - FLUORESCENT PENETRANT INSPECTION |  |  | EXECUTAR FPI: ASTM E 1417, METHOD-A, LEVEL-3. APRESENTOU INDICAÇÃO? ( )SIM ( )NÃO RUN FPI: ASTM E 1417, METHOD-A, LEVEL-3. DID YOU PRESENT AN INDICATION? ( )YES ( )NO |  |  |  |  |  |  |
| 6 | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | LIMPAR CONFORME A IP-0006, 3.10 CLEAR AS PER IP-0006, 3.10 |  |  |  |  |  |  |
| 7 | REPAIR SHOP | ISOLAÇÃO / MASK |  |  | ISOLAR OS PRISIONEIROS, FUROS E AS ÁREAS DE CONTATO. ISOLATE STUDS, HOLES, AND CONTACT AREAS. |  |  |  |  |  |  |
| 8 | REPAIR SHOP | PINTURA / PAINTING |  |  | PINTAR EXTERNAMENTE COM: PRIMER PA-0060 e CINZA PA-0065 PAINT EXTERNALLY WITH: PRIMER PA-0060 and GREY PA-0065 |  |  |  |  |  |  |
| 9 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | REMOVER AS ISOLAÇÕES E OS EXCESSOS. REMOVE INSULATION AND EXCESSES. |  |  |  |  |  |  |
| 10 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | INSTALAR OS CAGES E FRENA-LOS COM PINOS P/N= 17341 INSTALL THE CAGES AND BRAKE THEM WITH P/N PINS = 17341 |  |  |  |  |  |  |
| 11 | T56A | INSPEÇÃO / INSPECTION |  |  | INSPEÇÃO FINAL. ( )APROVADO ( )REPROVADO FINAL INSPECTION. ( )APPROVED ( )FAILED |  |  |  |  |  |  |
| OBSERVAÇÕES (Observation): |  |  |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA 20/10/2025 (Date): |  |
|  |  |  |  |  |  | VILSON BARBOSA |  |  |  | 20/10/2025 |  |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department)