| PN (PN): |  |  | SN: |  |  |  |  | Nº da OS (WO): |  |  | TLN:02188 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6852307 |  |  | 894 |  |  |  |  | IAS |  |  |  |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  |  |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço PrincipaL (Main Service) |  | QUANT: |  |  |
| AE- 109008 |  |  |  | PUMP COVER |  |  | REVISÃO GERAL / OVERHAUL |  | 1 |  |  |
| ITEM (Item) | TAREFA (Task) | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |  |  |  |  |
| 1 | NDT | END- LIQUIDO PENETRANTE / NDT - FLUORESCENT PENETRANT INSPECTION |  |  | EXECUTAR FPI: ASTM E 1417, METHOD-A, LEVEL-3. APRESENTOU INDICAÇÃO? ( )SIM ( )NÃO RUN FPI: ASTM E 1417, METHOD-A, LEVEL-3. DID YOU PRESENT AN INDICATION? ( )YES ( )NO |  |  |  |  |  |  |
| 2 | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | LIMPAR CONFORME A IP-0006, 3.10 CLEAR AS PER IP-0006, 3.10 |  |  |  |  |  |  |
| 3 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | MONTAR A BUCHA P/N=6871625 E FRENA-LA COM O PINO P/N= AN150306. FIT THE CHUCK P/N=6871625 AND BRAKE IT WITH THE P/N= AN150306 PIN. |  |  |  |  |  |  |
| 4 | CHEMICAL CLEANING | USINAGEM / MACHINING |  |  | USINAR O DIÂMETRO INTERNO DA BUCHA, DEIXANDO COM 0.5007¨+ou- 0.0002¨ MACHINE THE INNER DIAMETER OF THE BUSHING, LEAVING IT WITH 0.5007 ̈+or- 0.0002 |  |  |  |  |  |  |
| 5 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | REMOVER PEDAÇO DA BUCHA PARA EXTENDER O RASGO DA FACE E FURAR A BUCHA PARA EXTENDER O FURO QUE VEM DA FACE. REMOVE PIECE OF THE BUSHING TO EXTEND THE TEAR FROM THE FACE AND DRILL THE BUSHING TO EXTEND THE HOLE THAT COMES FROM THE FACE. |  |  |  |  |  |  |
| 6 | REPAIR SHOP | ISOLAÇÃO / MASK |  |  | ISOLAR OS FUROS, ROSCA E AS ÁREAS DE CONTATO. ISOLATE THE HOLES, THREAD AND CONTACT AREAS. |  |  |  |  |  |  |
| 7 | REPAIR SHOP | PINTURA / PAINTING |  |  | PINTAR EXTERNAMENTE COM: PRIMER PA- 0060 CINZA PA- 0065 PAINT EXTERNALLY WITH: PRIMER PA- 0060 GREY PA-0065 |  |  |  |  |  |  |
| 8 | REPAIR SHOP | REMOVER ISOLAMENTO / REMOVE MASK |  |  | REMOVER AS ISOLAÇÕES E EXCESSOS DE TINTA. REMOVE INSULATION AND EXCESS PAINT. |  |  |  |  |  |  |
| 9 | T56A | INSPEÇÃO / INSPECTION |  |  | INSPEÇÃO FINAL. ( )APROVADO ( )REPROVADO FINAL INSPECTION. ( )APPROVED ( )FAILED |  |  |  |  |  |  |
| OBSERVAÇÕES (Observation): |  |  |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA 05/11/2025 (Date): |  |
|  |  |  |  |  |  | VILSON BARBOSA |  |  |  | 05/11/2025 |  |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department)