| PN (PN): |  | SN: |  |  | Nº da OS (WO): |  | TLN: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3041511 |  | A001WAE4 |  |  | 048866 |  | 01500 |  |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço Principal (Main Service): |  | QUANT: |  |
| PCE-AA0038 |  | HIGH PRESSURE TURBINE DISK |  |  | MONTAGEM / ASSEMBLY |  | 1 |  |
| TSN: N/A |  | CSN: N/A |  |  | TSO: N/A |  | CSO: N/A |  |
| ITEM (Item) | TAREFA (Task) |  | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |
| 01 | CTM |  | ANALISE INICIAL DE DOCUMENTAÇÃO / INITIAL DOCUMENTATION ANALYSIS |  |  | Verificar se o escopo está em conformidade com a revisão mais recente do manual aplicável. Para isso, conferir se a revisão mencionada abaixo corresponde à revisão vigente do manual disponível, conforme localização mencionada na Intranet: Conforme Overhaul Manual_3034623 _ Rev. 42, 13 Maio 2024. (Verify that the scope complies with the latest revision of the applicable manual. To do so, check whether the revision indicated below matches the current revision of the manual available, as per the location specified on the Intranet:) (According to Overhaul Manual_3034623 _ Rev. 42, 13 May 2024.) Nota: caso haja divergência, não seguir com as próximas etapas e notificar de forma imediata via e-mail a Engenharia para que seja realizada a atualização do escopo. (Note: if there is any discrepancy, do not proceed with the following steps and immediately notify the responsible Engineering team via email so that the scope can be updated accordingly.) |  |  |
| 02 | PW100 |  | INSPEÇÃO / INSPECTION |  |  | Realizar a análise e marcação da posição do trim na Front Cover e Interstage Air Seal conforme Overhaul Manual 3034623, ATA 72-50-10, Seção “Subassembly-2”, Parágrafo 10., Etapa E., Figura 522 (Trim Balancing of HP Rotor Assembly) (Perform the analysis and marking of the trim position on the Front Cover and Interstage Air Seal in accordance with Overhaul Manual 3034623, ATA 72-50-10, Section “Subassembly-2,” Paragraph 10, Step E, Figure 522 (Trim Balancing of HP Rotor Assembly).) Pesos de Balanceamento Fino da Front Cover: (Front Cover trim balancing weights:) ____________ ____________ ____________ ____________ ____________ ____________ ____________ ____________ Pesos de Balanceamento Fino do Interstage Air Seal: (Interstage Air Seal trim balancing weights:) ____________ ____________ ____________ ____________ ____________ ____________ ____________ ____________ |  |  |
| 03 | PW100 |  | INSTALAR / INSTALL |  |  | Realizar a instalação da Front Cover, Rear Cover e Interstage Air Seal conforme Overhaul Manual 3034623, ATA 72-50-10, Seção “Subassembly-2”, Parágrafo 9., Etapa D., Figura 514 (Front and Rear Cover Installation): (Install the Front Cover, Rear Cover, and Interstage Air Seal in accordance with Overhaul Manual 3034623, ATA 72-50-10, Section “Subassembly-2,” Paragraph 9., Step D., Figure 514 (Front and Rear Cover Installation).) PN da Front Cover Instalada: _____________ (PN of Installed Front Cover:) SN da Front Cover Instalada: _____________ (SN of Installed Front Cover:) PN da Rear Cover Instalada: _____________ (PN of Installed Rear Cover:) SN da Rear Cover Instalada: _____________ (SN of Installed Rear Cover:) PN do Interstage Air Seal Instalado: _____________ (PN of Installed Interstage Air Seal:) SN do Interstage Air Seal Instalado: _____________ (SN of Installed Interstage Air Seal:) |  |  |
| 04 | PW100 |  | REMOÇÃO / REMOVE |  |  | Realizar a remoção dos trim na Front Cover e Interstage Air Seal conforme Overhaul Manual 3034623, ATA 72-50-10, Seção “Subassembly-2”, Parágrafo 10., Etapa E., Figura 522 (Trim Balancing of HP Rotor Assembly). (Perform the removal of the trim weights from the Front Cover and Interstage Air Seal in accordance with Overhaul Manual 3034623, ATA 72-50-10, Section “Subassembly-2,” Paragraph 10, Step E, Figure 522 (Trim Balancing of HP Rotor Assembly).) |  |  |
| 05 | PW100 |  | BALANCEAMENTO / BALANCING |  |  | Realizar o balanceamento do conjunto de HP Disk conforme Overhaul Manual 3034623, ATA 72-50-10, Seção “Subassembly-2”, Parágrafo 9., Etapa G., Figura 517 (Detail Balancing of HP Turbine Disk Assembly (Method A) (Post-SB20918/Post-SB21321)) (Perform the balancing of the HP Disk assembly in accordance with Overhaul Manual 3034623, ATA 72-50-10, Section “Subassembly-2,” Paragraph 9, Step G, Figure 517 (Detail Balancing of HP Turbine Disk Assembly (Method A) (Post-SB20918/Post-SB21321)).) (ref. Parágrafo G (1)) (ref. Paragraph G (1)) NCC:_______ Medida do Diâmetro A: __________ (Measured Diameter A:) (ref. Parágrafo G (9)) (ref. Paragraph G (9)) NCC:_______ Desbalanceamento inicial no Plano C: __________ [g.mm] (Initial unbalance in Plane C:) Ângulo do desbalanceamento inicial no Plano C: _______ [°] (Initial unbalance angle in Plane C:) Desbalanceamento inicial no Plano D: __________ [g.mm] (Initial unbalance in Plane D:) Ângulo do desbalanceamento inicial no Plano D: _______ [°] (Initial unbalance angle in Plane D:) (ref. Parágrafo G (10)) (ref. Paragraph G (10)) NCC:_______ Desbalanceamento final no Plano C: __________ [g.mm] (Final unbalance in Plane C:) Ângulo do desbalanceamento final no Plano C: _______ [°] (Final unbalance angle in Plane C:) Desbalanceamento final no Plano D: __________ [g.mm] (Final unbalance in Plane D:) Ângulo do desbalanceamento final no Plano D: _______ [°] (Final unbalance angle in Plane D:) |  |  |
| 06 | PW100 |  | REMOÇÃO / REMOVE |  |  | Realizar a instalação dos trims no Front Cover e Interstage Air Seal conforme Overhaul Manual 3034623, ATA 72-50-10, Seção “Subassembly-2”, Parágrafo 10., Etapa E., Figura 522 (Trim Balancing of HP Rotor Assembly). (Perform the installation of the trims on the Front Cover and Interstage Air Seal in accordance with Overhaul Manual 3034623, ATA 72-50-10, Section “Subassembly-2,” Paragraph 10, Step E, Figure 522 (Trim Balancing of HP Rotor Assembly).) |  |  |
| 07 | PW100 |  | INSPEÇÃO / INSPECTION |  |  | Inspeção final. (Final Inspection.) |  |  |
| OBSERVAÇÕES (Observation): |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA (Date): |
|  |  |  |  | GUILHERME BRAGA |  |  |  | 10/11/2025 |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department.)