| PN (PN): |  |  | SN: |  |  |  |  | Nº da OS (WO): |  |  | TLN: 01153 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23006940 |  |  | SA56443A |  |  |  |  | 050700 |  |  |  |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  |  |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço PrincipaL (Main Service) |  | QUANT: |  |  |
| COMPRESSOR- ADOC 702131- EQUADOR |  |  |  | ACCESSORY BOX HOUSING |  |  | REVISÃO GERAL / OVERHAUL |  | 1 |  |  |
| ITEM (Item) | TAREFA (Task) | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |  |  |  |  |
| 1 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | REMOVER OS 3- CAGES CENTRAIS. NOTA: PEDIR PARA O CONTROLE ADIANTAR A ENTREGA DOS 3- CAGES PARA A INSPEÇÃO, ENCAMINHA-LOS DE IMEDIATO PARA CROMO. REMOVE THE 3-CENTER CAGES. NOTE: ASK THE CONTROL TO ADVANCE THE DELIVERY OF THE 3- CAGES FOR INSPECTION, IMMEDIATELY FORWARD THEM TO CHROME. |  |  |  |  |  |  |
| 2 | REPAIR SHOP | USINAGEM / MACHINING |  |  | USINAR O DIÂMETRO ¨B¨, CENTRANDO PELO DIÂMETRO ¨A¨, APENAS PARA SERVIR DE REFERÊNCIA, REMOVENDO O MÍNIMO POSSÍVEL PARA LIMPA-LO. VEJA A FIGURA 3- ANEXA. MACHINE THE DIAMETER ̈B ̈, CENTERING BY THE DIAMETER ̈A ̈, JUST TO SERVE AS A REFERENCE, REMOVING AS LITTLE AS POSSIBLE TO CLEAN IT. SEE FIGURE 3 - ATTACHED. |  |  |  |  |  |  |
| 3 | REPAIR SHOP | USINAGEM / MACHINING |  |  | USINAR O DIÂMETRO ¨C¨, DEIXANDO CENTRADO COM O ¨B¨ E COM 1.940¨+ou- 0.003¨ MACHINING THE DIAMETER ̈C ̈, LEAVING CENTERED WITH THE ̈B ̈ AND WITH 1.940 ̈ + OR - 0.003 |  |  |  |  |  |  |
| 4 | REPAIR SHOP | USINAGEM / MACHINING |  |  | USINAR O ALOJAMENTO INTERNO DO CAGE CENTRAL, CENTRANDO PELO DIÂMETRO ¨B¨ NOTA: REMOVER O MÍNIMO POSSÍVEL PARA DESOVALIZA-LO. MACHINING THE INNER HOUSING OF THE CENTRAL CAGE, CENTERING BY DIAMETER ̈B ̈ NOTE: REMOVE AS LITTLE AS POSSIBLE TO DISPOSE OF IT. |  |  |  |  |  |  |
| 5 | REPAIR SHOP | USINAGEM / MACHINING |  |  | USINAR O ALOJAMENTO DOS 2- CAGES ADJACENTES AO CENTRAL. REMOVENDO O MÍNIMO POSSÍVEL PARA DESOVALIZA-LOS. MACHINE THE HOUSING OF THE 2-CAGES ADJACENT TO THE CENTRAL ONE. REMOVING AS LITTLE AS POSSIBLE TO DE-EVALUATE THEM. |  |  |  |  |  |  |
| 6 | REPAIR SHOP | USINAGEM / MACHINING |  |  | CONFECCIONAR UMA BUCHA DE ALUMÍNIO, CONFORME A FIGURA 3- ANEXA, DETALHE-A. DEIXANDO O DIÂMETRO EXTERNO COM UM APERTO NO DIÂMETRO ¨C¨= 0.0025¨+ou-0.0005¨ MAKE AN ALUMINUM BUSHING, AS SHOWN IN FIGURE 3 - ATTACHED, DETAIL IT. LEAVING THE OUTSIDE DIAMETER WITH A GRIP ON DIAMETER ̈C ̈= 0.0025 ̈+or -0.0005 |  |  |  |  |  |  |
| 7 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | MONTAR A BUCHA CONFECCIONADA E FRENA-LA COM O PINO P/N=17341 VEJA A FIGURA 3- ANEXA. ASSEMBLE THE MADE BUSHING AND BRAKE IT WITH THE PIN P/N=17341 SEE FIGURE 3 - ATTACHED. |  |  |  |  |  |  |
| 8 | REPAIR SHOP | USINAGEM / MACHINING |  |  | USINAR A BUCHA NOVA, DEIXANDO PLANA COM A FACE E COM O DIÂMETRO INTERNO= 1.498¨+0.001¨ DEIXAR CENTRADO COM OS DIÂMETROS: A e B. MACHINE THE NEW BUSHING, LEAVING IT FLAT WITH THE FACE AND WITH THE INTERNAL DIAMETER= 1.498 ̈+0.001 ̈ LEAVE CENTERED WITH DIAMETERS: A and B. |  |  |  |  |  |  |
| 9 | REPAIR SHOP | REPAROS MANUAIS / MANUAL REPAIR |  |  | MONTAR OS 3- CAGES (1, o 2-CENTRAL e o 3) FRENA-LOS COM PINOS P/N=17341. ASSEMBLE THE 3-CAGES (1, 2-CENTRAL and 3) BRAKE THEM WITH PINS P/N=17341. |  |  |  |  |  |  |
| 10 | REPAIR SHOP | USINAGEM / MACHINING |  |  | USINAR O ENCAIXE EXTERNO COM ROSCA ASSINALADO, AUMENTANDO PARA AS MEDIDAS DA FIGURA 2- ANEXA, DETALHE-A. MACHINE THE EXTERNAL FITTING WITH THE INDICATED THREAD, INCREASING TO THE MEASUREMENTS OF FIGURE 2- ATTACHED, DETAIL IT. |  |  |  |  |  |  |
| 11 | REPAIR SHOP | ISOLAÇÃO / MASK |  |  | ISOLAR OS PRISIONEIRO, OS FUROS, A PLACA DE IDENTIFICAÇÃO E AS ÁREAS DE CONTATO. ISOLATE THE STUDS, HOLES, NAMEPLATE, AND CONTACT AREAS. |  |  |  |  |  |  |
| 12 | REPAIR SHOP | PINTURA / PAINTING |  |  | PINTAR EXTERNAMENTE COM: PRIMER PA-0060 e CINZA PA-0065. PAINT EXTERNALLY WITH: PRIMER PA-0060 and GRAY PA-0065. |  |  |  |  |  |  |
| 13 | REPAIR SHOP | REMOVER ISOLAMENTO / REMOVE MASK |  |  | REMOVER AS ISOLAÇÕES E OS EXCESSOS DE TINTA. REMOVE INSULATION AND EXCESS PAINT. |  |  |  |  |  |  |
| 14 | CHEMICAL CLEANING | DEPOSIÇÃO DE ANODIZAÇÃO / ANODIZING PLATING |  |  | NEUTRALIZAR A ENTRADA COM ROSCA EXTERNA USINADA, (MAGNÉSIO) NEUTRALIZE THE INPUT WITH MACHINED EXTERNAL THREAD, (MAGNESIUM) |  |  |  |  |  |  |
| 15 | CHEMICAL CLEANING | LIMPAR / CLEAN |  |  | LIMPAR CONFORME A IP-0006, 3.10. CLEAR AS PER IP-0006, 3.10. |  |  |  |  |  |  |
| 16 | T56A | INSPEÇÃO / INSPECTION |  |  | INSPEÇÃO FINAL. FINAL INSPECTION. |  |  |  |  |  |  |
| OBSERVAÇÕES (Observation): |  |  |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA 08/06/2026 (Date): |  |
|  |  |  |  |  |  | VILSON BARBOSA |  |  |  | 08/06/2026 |  |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department)