| PN (PN): |  | SN: |  |  | Nº da OS (WO): |  | TLN: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3108979-01 |  | N-C |  |  | 055040 |  | 01497 |  |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço Principal (Main Service): |  | QUANT: |  |
| PCE-AX0093 |  | TWO-HOLE ONE-GROOVE PACKING COVER |  |  | INSPEÇÃO / INSPECTION |  | 1 |  |
| TSN: N/A |  | CSN: N/A |  |  | TSO: N/A |  | CSO: N/A |  |
| ITEM (Item) | TAREFA (Task) |  | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |
| 01 | CTM |  | ANALISE INICIAL DE DOCUMENTAÇÃO / INITIAL DOCUMENTATION ANALYSIS |  |  | Verificar se o escopo está em conformidade com a revisão mais recente do manual aplicável. Para isso, conferir se a revisão mencionada abaixo corresponde à revisão vigente do manual disponível, conforme localização mencionada na Intranet: Conforme PW100 SERIES Cleaning, Inspection, Repair Manual_3043515 _ Rev. 51, 13 Abril. 2026. (Verify that the scope complies with the latest revision of the applicable manual. To do so, check whether the revision indicated below matches the current revision of the manual available, as per the location specified on the Intranet:) (According to PW100 SERIES Cleaning, Inspection, Repair Manual_3043515 _ Rev. 51, 13 April. 2026) Nota: caso haja divergência, não seguir com as próximas etapas e notificar de forma imediata via e-mail a Engenharia para que seja realizada a atualização do escopo. (Note: if there is any discrepancy, do not proceed with the following steps and immediately notify the responsible Engineering team via email so that the scope can be updated accordingly.) |  |  |
| 02 | PW100 |  | INSPEÇÃO / INSPECTION |  |  | Realizar a inspeção visual da peça conforme CIR Manual 3043515, ATA 72-11-91, Seção “Inspection/Check-02”, Parágrafo 1., Etapa C.; Figura 801, Tabela 801 (Task 72-11-91-220-801). (Perform the visual inspection of the part in accordance with CIR Manual 3043515, ATA 72-11-91, Section 'Inspection/Check-02', Paragraph 1., Step C.; Figure 801 and Table 801 (Task 72-11-91-220-801).) A peça foi aprovada na inspeção? (Was the part approved in the inspection?) ( ) Sim, aprovada. (Yes, approved.) ( ) Reparos aplicáveis. Nº da sub O.S. aberta para realização da etapa: ____________________ (Applicable repairs. Number of the sub W.O. opened for the performance of the step:)) ( ) Não, rejeitada. (No, rejected.) |  |  |
| 03 | PW100 |  | INSPEÇÃO / INSPECTION |  |  | Inspeção final. (Final Inspection.) |  |  |
| OBSERVAÇÕES (Observation): |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA (Date): |
|  |  |  |  | GUILHERME BRAGA |  |  |  | 11/08/2026 |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department.)