| PN (PN): |  | SN: |  |  | Nº da OS (WO): |  | TLN: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3124135-01 |  | N-C |  |  | 052155 |  | 01509 |  |
| Nº DO ARTIGO PRINCIPAL (Main Article): |  | ARTIGO/PARTE (Article/Part): |  |  | Serviço Principal (Main Service): |  | QUANT: |  |
| PCE-AX0090 |  | GAS GENERATOR CASE ASSY |  |  | LIMPEZA |  | 1 |  |
| TSN: N/A |  | CSN: N/A |  |  | TSO: N/A |  | CSO: N/A |  |
| ITEM (Item) | TAREFA (Task) |  | ETAPA (Step) |  |  | DESCRIÇÃO (Description) |  |  |
| 01 | CTM |  | ANALISE INICIAL DE DOCUMENTAÇÃO / INITIAL DOCUMENTATION ANALYSIS |  |  | Verificar se o escopo está em conformidade com as revisões mais recentes das publicações aplicáveis. Para isso, conferir se as revisões mencionadas abaixo correspondem as revisões vigentes das publicações disponíveis, conforme localização mencionada na Intranet: Conforme Maintenance Manual _ 3044822 _ PW127G Engines _ Rev. 56, 15 Jun. 2026. (Verify whether the scope is in accordance with the latest revisions of the applicable publications. To do so, check if the revisions mentioned below correspond to the current revisions of the available publications, as indicated in the location specified on the Intranet:) (According to Maintenance Manual _ 3044822 _ PW127G Engines _ Rev. 56, 15 Jun. 2026.) Nota: Caso haja divergência, não seguir com as próximas etapas e notificar de forma imediata via e-mail a Engenharia para que seja realizada a atualização do escopo. (Note: If there is any discrepancy, do not proceed with the following steps and immediately notify the responsible Engineering team via email so that the scope can be updated accordingly.) |  |  |
| 02 | CHEMICAL CLEANING |  | LIMPAR / CLEAN |  |  | Realizar a limpeza das peças, conforme Maintenance Manual 3044822, ATA 72-40-00, Seção “Cleaning/Painting”, Parágrafo 7., Etapa A. e Seção “Consumable Materials”, Tabela 1. (Perform the cleaning of the parts, in accordance with Maintenance Manual 3044822, ATA 72-40-00, Section “Cleaning/Painting”, Paragraph 7., Step A. and Section “Consumable Materials”, Table 1. A menos que seja especificado de outra forma, limpe todas as peças usando solvente de petróleo (PWC11-027) ou produto de limpeza (PWC11-031). (Unless otherwise specified, clean all parts, using petroleum solvent (PWC11-027) or cleaner (PWC11-031).) |  |  |
| 03 | PW100 |  | INSPEÇÃO / INSPECTION |  |  | Realizar a inspeção final da peça. (Perform the final inspection of the part.) |  |  |
| OBSERVAÇÕES (REMARKS): |  |  |  | ASSINATURA DO SOLICITANTE (Signature): |  |  |  | DATA (Date): |
| - |  |  |  | GUILHERME BRAGA |  |  |  | 21/08/2026 |

- Em caso de serviço externo, o fornecedor deverá ser indicado pelo setor de compras.

(In case of external service, the supplier must be indicated by the purchasing department.)